| Заказчик/Customer |  |  |  |  | Aeroflot |  | Номер заказа/№ пп / RO/Item № |  |  |  |  | R49069626 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Черт. номер компонента:/ P/N: |  |  |  |  | D2557216001800 |  | Карта-наряд / WSS |  |  |  |  |  |  |  |  |
| Сер. или внутр. номер:/ S/N or internal No: |  |  |  |  | 101500324 |  | Плановая трудоёмкость Planned MHRS |  |  |  |  | 11 MHRS |  |  |  |
| Статус/Вид работ Status/Work |  |  | Инспекция/проверка Inspection/Check ☒ |  |  | Ремонт / Repair ☐ |  |  |  |  | Капремонт / Overhaul ☐ |  |  | Модификация / Modification ☐ |  |
| Подготовил/Prepared by: |  |  |  |  |  |  |  |  | Утвердил/Approved by |  |  |  |  |  |  |
| Дата/Date |  |  |  | ФИО/Name |  |  |  |  | Дата/Date |  |  |  | ФИО/Name |  |  |
| 17.02.2026 |  |  |  | I. Beliakov |  |  |  |  | 17.02.2026 |  |  |  | S. Gorbachev |  |  |
| Детали и материалы / Parts and Materials: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No | Описание / Description |  |  |  |  |  |  | Чертёжный номер / Part Number |  |  |  |  |  |  | Кол-во / Qty |
|  | Solvent (MEK, Isopropyl Alcohol 99% or equivalent) |  |  |  |  |  |  | N/A |  |  |  |  |  |  | A/R |
|  | Textile-Lint free cotton |  |  |  |  |  |  | 14SBA1 or equivalent |  |  |  |  |  |  | A/R |
| Оборудование и инструменты / Equipment & Tools: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No | Описание / Description |  |  |  |  |  |  |  |  | Чертёжный номер / Part Number |  |  |  |  |  |
|  | NDT Tool |  |  |  |  |  |  |  |  | Olympus 600 |  |  |  |  |  |
| Ссылка на ЭД / Doc. reference: |  | AIRBUS A320: CMM 25-59-63, NTM 51-10-08. |  |  |  |  |  |  |  |  |  |  |  |  |  |

# -

| Технология выполнения / Accomplishment instructions |  |  |  |
| --- | --- | --- | --- |
| ОЧИСТКА / CLEANING |  |  |  |
| Item | IAW CMM TASK 25-59-63-100-801-A01 | Исп./ Mech. | Инсп./ Insp. |
| Refer to 25-59-63-100-001-A01 Cleaning of the Painted Component Parts A. Cleaning of the Painted Component Parts (1) Remove the loose dirt with a vacuum cleaner or a soft brush. (2) Remove the remaining sealant and dirt with a nonmetallic scraper. (3) Clean the component parts with Solvent and a soft brush or the Material No. 14SBA1 Textile-Lint free Cotton. (4) Remove the detergent with clean water. (5) Dry the component parts with a clean, dry Material No. 14SBA1 Textile-Lint free Cotton - or a hot-air blower. |  |  |  |
| Item | IAW CMM TASK 25-59-63-100-802-A01 | Исп./ Mech. | Инсп./ Insp. |
| Refer to Subtask 25-59-63-100-002-A01 Cleaning of the Component Parts which are not Painted. A. Cleaning of the Component Parts which are not Painted (1) Remove the loose dirt with a vacuum cleaner or a soft brush. (2) Remove the remaining dirt with a nonmetallic scraper. (3) Clean the component parts with the Material No. 08BBF1 Non Aqueous Cleaner- Methyl-Ethyl-Ketone - and a soft brush or the Material No. 14SBA1 Textile-Lint free Cotton. (4) Dry the component parts with a clean, dry Material No. 14SBA1 Textile-Lint free Cotton - or a hot-air blower. |  |  |  |
| ИНСПЕКЦИЯ / INSPECTION/CHECK |  |  |  |
| Item | IAW CMM TASK 25-59-63-010-801-A01 | Исп./ Mech. | Инсп./ Insp. |
| - Remove the Ball Units IAW CMM task 25-59-63-010-801-A01 and FIGURE 2 for inspection. - Put the pins of the Ball Units Wrench into the holes of the upper cover of the ball units. Refer to Figure 2. - Push the attachment pins against the spring pressure and remove the ball units from the ball mat/ball strip assembly. - Clean the ball units, if necessary. - Perform Visual Inspection open hole of place install ball unit. If found corrosion or other damage, mark all found deviations on Figure 1-7 and record details in Table 1. |  |  |  |
| Item | IAW CMM TASK 25-59-63-210-801-A01 | Исп./ Mech. | Инсп./ Insp. |
| Check of the Metal and GFRP Components of the Ball Mats for Cracks. NOTE: If you think there is a crack, you must do an approved nondestructive test. If a metal component has a crack: - Replace the component (See the DISASSEMBLY and ASSEMBLY sections of CMM 25-59-63). |  |  |  |
| Item | NDT Hight frequents eddy current inspection IAW NDT 51-10-08 | Исп./ Mech. | Инсп./ Insp. |
|  |  | NDT |  |
| If REQUIRED, perform NDT Hight frequents eddy current inspection Note: Perform HFECI for cracks only when suspected cracks area detected through visually inspection. Perform a Hight frequents eddy current inspection to confirmed crack refer to NTM 51-10-08 NDT report №:___________________________________ |  |  |  |
| Item | IAW CMM TASK 25-59-63-210-801-A01 | Исп./ Mech. | Инсп./ Insp. |
| Check of the Metal and GFRP Components of the Ball Mats for Scratches. 1 If the surface of the metal component has a scratch: a. Repair the scratch (See REPAIR TASK 25-59-63-300-802-A01). b. When the scratch is fully removed, measure the dimensions of the repair: -A maximum repair length of 25.0 mm (0.984 in) is permitted. -A maximum repair depth of 0.25 mm (0.0098 in) is permitted. 2 If the GFRP has a scratch: - Install a patch for damage that is more than 6.0 mm (0.2362 in), - see the REPAIR TASK 25-59-63-300-804-A01. - Replace the component (See REPAIR TASK 25-59-63-350-806-A01): - If the patch will overlap an adjacent repair - If the patch edge distance is less than 25.4 mm (1.0 in) |  |  |  |
| Item | IAW CMM TASK 25-59-63-210-801-A01 | Исп./ Mech. | Инсп./ Insp. |
| Check of the Metal and GFRP Components of the Ball Mats for Marks. 1 If the surface of the component has a mark: a. Repair the mark (See REPAIR TASK 25-59-63-300-801-A01). b. When the mark is fully removed, measure the dimensions of the repair: -A maximum repair depth of 0.25 mm (0.0098 in) is permitted. -A maximum repair area of 150.0 mm² (0.2325 in²) is permitted. c. Measure the distance between the repair and an adjacent repair (if applicable): -The distance between the edges of two adjacent repairs must be more than 50.0 mm (1.969 in). - Replace the components, if their repair dimensions or the distance between repairs are not in the limits (See the DISASSEMBLY and ASSEMBLY sections of CMM 25-59-63). |  |  |  |
| Item | IAW TASK 25-59-63-210-801-A01 | Исп./ Mech. | Инсп./ Insp. |
| Check of the Metal and GFRP Components of the Ball Mats for Corrosion. 1 If the metal component has corrosion: a. Repair the corrosion (See REPAIR TASK 25-59-63-300-801-A01). b. When the corrosion is fully removed, measure the maximum depth of the repair: -A maximum repair depth of 0.25 mm (0.0098 in) in all areas is permitted. -A maximum repair area of 150.0 mm² (0.2325 in²) is permitted. c. Replace the components, if their repair dimensions are not in the limits (See the DISASSEMBLY and ASSEMBLY sections of CMM 25-59-63). 2 If there corrosion between the GFRP and the metal component: - Install a patch for damage that is more than 6.0 mm (0.2362 in), - see the REPAIR TASK 25-59-63-300-804-A01of CMM 25-59-63. - Replace the component (See REPAIR TASK 25-59-63-350-806-A01 of CMM 25-59-63): - If the patch will overlap an adjacent repair - If the patch edge distance is less than 25.4 mm (1.0 in). |  |  |  |
| Item | IAW CMM TASK 25-59-63-210-801-A01 | Исп./ Mech. | Инсп./ Insp. |
| Check of the Metal and GFRP Components of the Ball Mats for Debonding. 1 If the GFRP component is debonded: a. Install a patch, see the REPAIR TASK 25-59-63-300-804-A01 of CMM 25-59-63. b. Replace the component (See REPAIR TASK 25-59-63-350-806-A01 of CMM 25-59-63): -If the patch will overlap an adjacent repair -If the patch edge distance is less than 25.4 mm (1.0 in). |  |  |  |
| Item | IAW CMM TASK 25-59-63-210-801-A01 | Исп./ Mech. | Инсп./ Insp. |
| Check of the Metal and GFRP Components of the Ball Mats for Damaged protective treatment. 1 If the protective treatment of the component is damaged: -Repair the protective treatment (See REPAIR TASK 25-59-63-300-803-A01 of CMM 25-59-63). |  |  |  |
| Item | Check of the Ball Units IAW CMM TASK 25-59-63-210-802-A01 | Исп./ Mech. | Инсп./ Insp. |
| - Examine the ball units for general condition. (2) Replace the ball units which are not in a good condition (see DISASSEMBLY TASK 25-59-63-010-801-A01 and ASSEMBLY TASK 25-59-63-410-801-A01 of CMM 25-59-63). |  |  |  |
| Item | Check of the Sealing Tape IAW CMM TASK 25-59-63-210-803-A01 | Исп./ Mech. | Инсп./ Insp. |
| (1) Examine the sealing tape for: - Debonding - Cracks - Oil contamination - Porosity - Hardness. (a) If a sealing tape has signs of debonding, cracks, oil contamination, porosity or hardness: - Replace the sealing tape (See REPAIR TASK 25-59-63-350-803-A01 of CMM 25-59-63). |  |  |  |
| Item | Check of the Captive Screw IAW CMM TASK 25-59-63-210-805-A01 | Исп./ Mech. | Инсп./ Insp. |
| (1) Examine the captive screw for: - Operation mode - Corrosion. (a) Operation Mode 1 If a captive screw has no operation mode: a) Replace the defective captive screw (See REPAIR TASK 25-59-63-350-801-A01 of CMM 25-59-63). (b) Corrosion 1 If a captive screw has corrosion: a) Replace the defective captive screw (See REPAIR TASK 25-59-63-350-801-A01 of CMM 25-59-63). |  |  |  |
| Item | Result IAW CMM TASK 25-59-63-210-801-A01 | Исп./ Mech. | Инсп./ Insp. |
| 14. |  |  |  |
| Are any damages found? YES________/ NO_______ If damages found - Mark all found deviations on Figure 1 and record details in Table 1. |  |  |  |
| ЗАКЛЮЧИТЕЛЬНЫЕ ДЕЙСТВИЯ / FINAL ACTION |  |  |  |
| Item | Close up the inspection card | Исп./ Mech. | Инсп./ Insp. |
| 15. |  |  |  |
| Specify a revision of manual: CMM 25-59-63, Rev.: ____ Date:_______________ 2. (If Required) remarks: |

Технология выполнения / Accomplishment instructions

Фигура 1/Figure 1 Определение зоны / Zone definition Figure 08G / CMM 25-59-63-950-808-G01 Фигура 2/Figure 2 CMM FIGURE 25-59-63-991-004-A01 Фигура 3/Figure 3 Ball Mat P/N D2557216001800 Figure 72 / CMM 25-59-63-950-872-01

| Таблица/Table 1 Обнаруженные неисправности / Finding List |  |  |  |
| --- | --- | --- | --- |
| № позиции на рисунке/ Item # on Figure 1 | Описание неисправности/ повреждения / Description finding /damage | Размеры неисправности/ повреждения (Длина, Ширина, Глубина) / Damage size (Length Width Depth) (in or mm) | Возможность устранения неисправности/повреждения согласно применимой ЭД (да/нет) / Can be rectification iaw maintenance data (yes/no) |

| Фактическая трудоемкость/ Actual MHRS |  | Все работы выполнены/All tasks accomplished |  |  |
| --- | --- | --- | --- | --- |
|  |  | Дата/Date | ФИО/Name | Подпись и ID или штамп / Signature & ID number or stamp |